## PUMP MAINTENANCE SCHEDULE AND CHECKLISTS

Providing a maintenance schedule defined specifically by run hours or yardage pumped serves only as a general guideline given the large amount of variables a unit might be subject to, such as weather and ambient temperature conditions, jobsite conditions, material differences of concrete mix design, the load burden the unit is typically subject to, i.e. light, medium or heavy duty operation, etc.

The list that follows is to be used as a reference guide. However, the end user is encouraged to develop a preventative maintenance program that specifically suites their needs depending on the usage of the equipment.

| DAILY PUMP MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 1 | Check engine fluid levels (see engine manufacturer maintenance schedule for additional requirements) |  |
| 2 | Check hydraulic tank fluid levels |  |
| 3 | Check flush box fluid levels |  |
| 4 | Check condition of hydraulic oil for water or other contamination |  |
|  | Additional Notes: |  |
| 5 | Drain water from tank |  |
| 6 | Check hydraulic filter indicator condition |  |
| 7 | Lubricate lubrication points, during and after pumping |  |
| 8 | Inspect unit for fluid leaks, loose hoses, loose nuts, bolts, fasteners etc. |  |
|  | Additional Notes: |  |
| 9 | Trailer Towing Safety Inspection a) brakes functional |  |
|  | b) air pressure in tires is adequate and tire condition |  |
|  | c) all “lug nuts” are secure and in place |  |
|  | d) tow hitch is secure and in good condition |  |
| 10 | Additional Notes: |  |

| INITIAL 50 HOUR MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 1 | Change engine oil and filters (see engine manufacturer maintenance schedule for additional requirements) |  |
|  | Additional Notes: |  |
| 2 | Change hydraulic oil filters |  |
| 3 | Inspect hydraulic hoses and fittings for any signs of external wear or damage |  |
|  | Additional Notes: |  |

| EVERY 100 HOUR MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 1 | Clean hydraulic tank breather |  |
| 2 | Inspect all structural components (check frame, hopper, axle, hood, towing hook, and other structural members for any damage) |  |
|  | Additional Notes: |  |
| 3 | Adjust “S-Tube Nut” or “Castle Nut” (refer to “S-Tube Adjustment” in Maintenance Section) |  |
|  | Additional Notes: |  |
| 4 | Change flush box oil and examine for excessive amounts of contamination (excessive contamination might indicate need to change the Piston Cups) |  |
|  | Additional Notes: |  |
| 5 | Check coolers and radiators for dirt or debris. Clean as necessary |  |
| 6 | Check condition of engine drive belts. Change if necessary (see engine manufacturer maintenance schedule for additional requirements) |  |

| EVERY 250 HOUR MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 1 | Change hydraulic filters (or change more frequently as indicated by indicator gauge on filter) |  |
| 2 | Check that S-Tube, wear parts and seals in hopper are secure and adjusted well, rotate wear ring and replace seal if necessary |  |
|  | Additional Notes: |  |
| 3 | Check swing cylinder components: cylinders, bell crank, pins, bushings, bearings and grease fittings are secure, tight, and not worn excessively |  |
|  | Additional Notes: |  |
| 4 | Check piston cup wear (as indicated by analysis of contaminants found in flush box oil) |  |
|  | Additional Notes: |  |
| 5 | Check that all electrical wires, cables, terminals, plugs are in good condition |  |
|  | Additional Notes: |  |
| 6 | Change engine oil and filters (see engine manufacturer guide for all engine requirements) |  |
|  | Additional Notes: |  |
| 7 | Check condition of fuel hoses, fittings, and clamps |  |
|  | Additional Notes: |  |
| 8 | Inspect all safety decals to ensure that they are completely visible and legible |  |

| EVERY 250 HOUR MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 9 | Perform complete inspection of the controls |  |
| 10 | All toggles in good condition, stay in position or momentarily return to center |  |
| 11 | Control identification in good condition, legible |  |
| 12 | Gauges in good condition |  |
|  | Additional Notes: |  |
| 13 | Remote controls, control console in good condition |  |
| 14 | Switch in good condition |  |
| 15 | Cord in good condition, no cuts, securely mounted to box |  |
|  | Additional Notes: |  |
| 16 | Trailer frame integrity, visually check welds, cracks |  |
| 17 | Torsion axle secure |  |
| 18 | Wheels and tires, lug nuts tight, tire inflation |  |
| 19 | Electric brakes, breakaway switch connected |  |
| 20 | Front jack stand handle turns easily, smoothly |  |
| 21 | Manual jacks slide freely, lock pins in place |  |
| 22 | Lighting good condition operational |  |

| EVERY 500 HOUR MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 1 | Inspect hydraulic hoses and fittings for any signs of external wear or damage |  |
|  | Additional Notes: |  |
| 2 | Inspect all wear parts and change as necessary (excessive wear may cause inefficient performance and/or shutdown of operation) |  |
|  | Additional Notes: |  |
| 3 | Change hydraulic fluid, clean the reservoir and the suction strainers within reservoir, and replace all hydraulic oil filters |  |
|  | Additional Notes: |  |
| 4 | Complete inspection of the engine (refer to engine manufacturer maintenance schedule for details) |  |
| 5 | Inspect mountings, bolts, brackets |  |
| 6 | Oil level proper, coolant level proper, check for leaks |  |
| 7 | Fuel system, tank mounting, filter condition, check for leaks, damaged lines |  |
| 8 | Battery hold down, condition, tightness of cables |  |
| 9 | Key switch, indicator lights operable |  |
| 10 | Throttle control functional |  |
| 11 | Air cleaner and muffler securely mounted |  |
| Additional Notes: |  |  |

| EVERY 500 HOUR MAINTENANCE CHECKLIST |  |  |
| --- | --- | --- |
| Actual Hours Date / / |  |  |
| # | Maintenance Description | Initials |
| 12 | Pump cell check for structural damage, cracked welds |  |
| 13 | Hydraulic drive cylinders in good condition, secure, check for leaks |  |
| 14 | Material cylinders secure, tie rods tight |  |
| 15 | Water box structurally sound, clean, cover in place |  |
| 16 | S-Tube shift mechanism structurally sound, all pins and retainers in place |  |
| 17 | Hydraulic shift cylinders in good condition |  |
| 18 | Bearing housing, seals etc. in good condition |  |
| 19 | Hydraulic hoses secure no leaks |  |
|  | Additional Notes: |  |
| 20 | Hopper check for structural damage, cracked welds |  |
| 21 | S-Tube secure, in good condition |  |
| 22 | Check condition of wear plate, wear ring, seals |  |
| 23 | Check connection of S-Tube to outlet, seals, bearing |  |
| 24 | Hopper drain is functional |  |
| Additional Notes: |  |  |

EVERY 500 HOUR MAINTENANCE CHECKLIST

Actual Hours Date / /

ADDITIONAL GENERAL NOTES: